Paddle Boat Canopy Package Assembly

Required Materials:

- Regular Flat Head Screwdriver

- Tape Measure

- Canopy Package Complete

***DO NOT OVER TIGHTEN THE SET SCREWS!

- Lay out the canopy top so that the inside is facing up (tags are on the inside and at the rear).

- Insert the short bow into the middle rod pocket.

- Place the black canopy strap in the center of the cut out of the outside rod pockets with the rough sew seam facing up.

- Insert a long bow into each outside rod pocket and loose canopy straps.

- Fasten the Velcro around the bows at each corner of the canopy.

- Place 3 frame joints on the rear bow in this order making sure that all set screws on the side of the eye ends are facing toward you: 1 facing front, 1 facing rear, 1 facing front.

- Place one eye end completely on the bow with the set screw facing out and the tab perpendicular to the canopy bow.

- Tighten the set screw on the eye end until snug.

- Slide the top frame joint up snug to the eye end.

- With the set screw facing outward, make sure the joint is perpendicular to the canopy bow, then tighten the set screw until snug.

- With a tape measure, measure 17” from the flat part on the eye end to the middle of the bottom joint, making sure that the joint is perpendicular to the canopy bow.

- Tighten the set screw until snug.

- With a tape measure, measure 12” (PK3000) or 14 1/16” (PK4400) from the flat part on the eye end to the middle of second joint, making sure that the joint is perpendicular to the canopy bow.

- Tighten the set screw until snug.

- Repeat steps 6-12 on the other side of the same rear canopy bow.

- Insert the other long bow and short bow into the eye ends of the two joints facing forward.

- Tighten all set screws on eye ends until snug.

- The rear facing joint is for the support arms on your canopy. The PK3000 support arms are longer than the PK4400 ones.

- Insert the support arms into this joint and tighten the set screw until snug.